Memo

Date:		19th September 2023

To:		Jade Corcoran

Your Ref:	23/82059/DISCON

My Ref:	23/82059/DISCON

Subject:            2 Church Street, Eccles - (C8)

Proposal:	Request for confirmation of compliance of condition 8 (Vehicular Access) attached to planning permission 19/73148/FUL.

Location:	2 Church Street Eccles M30 0DF.

Condition 8 (Vehicular Access and External Highway Works)

Condition 8 states: No above ground development shall commence until full construction details of the proposed vehicular access, including pedestrian crossing provision(s) and the traffic light system, and works to John William Street, Birkdale Grove and Vicarage Grove as detailed on approved plan 2411-02/Rev C/Proposed General Arrangement Plan has been submitted to and approved in writing by the Local Planning Authority. The access arrangements shall be constructed as approved prior to first occupation and retained for the lifetime of the development.

With reference to drawings:

- 13352-BKP-00-XX-DR-C-0753-P7 – Site Clearance

- 13352-BKP-00-XX-DR-C-0754-P7 – GA

- 13352-BKP-00-XX-DR-C-0755-P8 – External Works

- 13352-BKP-00-XX-DR-C-0757-P6 – Drainage

- 13352-BKP-00-XX-DR-C-0758-P5 – Visibility Splay

- 13352-BKP-00-XX-DR-C-0759-P6 – Traffic Signs and Road Markings S278

- 13352-BKP-00-XX-DR-C-0760-P4 – Standard Details 1 Of 2

- 13352-BKP-00-XX-DR-C-0761-P5 – Standard Details 2 Of 2

- 010754-01-P01 – Proposed Street Lighting

Highways have raised issues with the above drawings and therefore cannot support the discharging of condition 8 (Vehicular Access and External Highway Works) attached to planning permission 19/73148/FUL. Please find comments raised below by both the Traffic and Transportation team:

- With regards to the works on Birkdale Road:

- The proposed pedestrian dropped crossing with tactile block paver must be relocated away from with the vicinity of the parking bay or parking bay shorten to accommodate the proposed pedestrian crossing point.

- The proposed 16m Loading bay can be shorten to 14m long and the extra 1m can be given over to general parking bay to increase it from 22m to 24m.

- The proposed desire line of the modified pedestrian crossing at the mouth of the John William Street / Birkdale Road is incorrect and should be parallel with the giveway road marking.

- Position of TRO signage and particularly the Loading only signage is located in the wrong position.

- With regards to the works on Vicarage Grove and Church Road:

- The proposed Loading only bay on Vicarage Grove needs to be supported by Loading only TRO.

- The position of the proposed pedestrian crossing point in relation to the ramp of the junction table needs to be reviewed and agreed with the council’s highway engineer.

- The existing cobble on the western side of the Church Street / Church Road junction needs to be taken up and resurface to prevent any trip hazards. This needs to be reviewed and agreed with the council’s highway engineer.

- In respect to the Traffic Signs and Road Marking Plan, the applicant needs to engage with the council’s Traffic Management team to agree a scheme and submit a plan that has been produced by the council’s Traffic Management team.

- With regards to the site access located off John William Street, the approved access at the building is approximately 3.25m wide at its narrowest point and therefore requires a traffic light system to manage traffic entering and exiting at entrance. This needs to be reviewed and a scheme needs to be developed to reduce conflict between entering and exiting traffic, submitted to comply with Condition 8.

Additional Note to Developer

Dilapidation Survey:

- Prior to ANY works commencing on site the developer shall contact John Horrocks/Pam Docksey to arrange a full dilapidation/Condition Survey of all adopted highways surrounding the site. Tel: 0161 603 4046/4006 or email: john.horrocks@salford.gov.uk or pam.docksey@salford.gov.uk

Highway Permits/Licensing:

- Applications for all forms of highway permits/licenses shall be made in advance of any works being undertaken on the adopted highway Note: NO boundary fencing shall be erected or positioned on any part of the adopted highway prior to first seeking the relevant permits/licenses from the Local Highway Authority Tel: 0161 603 4046/4006 or email: john.horrocks@salford.gov.uk or pam.docksey@salford.gov.uk

General Highway Information:

- Requests for general Information regarding the adopted highway network shall be directed to the Local Highway Authority – John Horrocks/Pam Docksey - 0161 603 4046 / 4006- Mobile: 07974157474 / 07925638521 Email: John.horrocks@salford.gov.uk  Pam.docksey@salford.gov.uk

S278 and S38 Works

- The Developer shall contact John Proctor to arrange for the S278 / S38 element of the development. Email: john.proctor@salford.gov.uk

Traffic Regulation Order and Traffic Management

- The Developer shall contact Michael Kilby or Judyta Hands to arrange for the TRO element of the development. Email: michael.kilby@salford.gov.uk or  judyta.hands@salford.gov.uk

- The Developer shall contact Barbara Grafham to arrange for the Temporary TRO and On-site Traffic Management element of the development. Email:  barbara.grafham@salford.gov.uk.

Kind regards,

Steve Ho

Highways Development Control

Email	steve.ho@salford.gov.uk