Memo

Date:		16th November 2021

To:		Danielle Sammy

Your Ref:	21/78661/FUL

My Ref:	21/78661/FUL

Subject:	266 Bury New Road

Proposal:	Construction of an infill building between two existing adjacent car showrooms in order to link them. In addition, a rejuvenation of the external facades, and internal reconfiguration, of the existing showrooms.

Location:	266 Bury New Road, Salford, M7 2YJ

Highway Comments:

The applicant is seeking permission to create a two storey infill building between two existing car dealerships which are amalgamating to become one dealership along with internal reconfiguration and changes to the facade. The site is located on Bury New Road in Broughton in a commercial area

The infill building that the applicant proposes to build will be 2 storeys high and will provide a link between the two existing building and change the existing façade of the building. The new building will contain some toilets, offices, parts storage, an after sales hub and a canteen on the first floor. As the applicant is proposing to increase the floor space on the site by 13.7m2 to a total of 130.9sqm2. It is anticipated that the extension is minor and unlikely to have any effect on the local highway network, to add to this the applicant has stated there will be no increase in the number of employees on site.

It should be noted that with the existing amenities within the building being relocated to the new link building, the ground floor layout will be redesigned. This will allow for the showroom to the north of the site to be slightly reduced in size, with a larger area being given over to parts storage and an increase in the associated workshop space. Currently the workshop space has room for 7 vehicles to be worked on at any one time. With the change in layout there is space for a total of 11 vehicles to be worked on at any one time. While this will potentially add more trips and usage associated with the building, as the site is located on a busy road in an existing commercial area, it is not thought that the increase in vehicles associated with the site will have a noticeable effect on the local highways.

It is stated in the application form that the development will decrease the on site parking by 7 spaces due to the new footprint of the building, this will still leave 153 spaces for use of the site, which is acceptable for the use of the building and the staff associated with the site and is unlikely to have a negative effect on the surrounding highways.

Recommendations:

The Local Highways Authority have no objections to the proposed planning application, due to the size and location of the proposed development.

Kind regards,

Louis Kennedy

Traffic and Transportation Apprentice

Email: Louis.Kennedy@Salford.gov.uk

Tel: 0161 779 6103