DELEGATED DECISION RECORD	REF: 22/80252/CLUDP

Address:	3 Stevenson Road, Swinton, Salford, M27 0SL

Proposal:	Application for lawful development certificate for a proposed single storey flat roof side extension with glazed roof lantern

Reason for delay beyond stat deadline and EoT:	N/A

Have all pre-commencement conditions been agreed with the applicant? N/A

Description of Site and Surrounding Area

This application relates to a semi-detached dwellinghouse located on Stevenson Road.

Description of Proposal

The applicant seeks a Certificate of Lawful Development for an erection of a single storey side extension with a flat roof and glazed roof lantern.

The extension would project a 3m from the main side wall of the original dwelling, measures 2.65m to the eaves and a maximum height of 3.45m.

## Publicity

Earliest Decision Date: N/A

Revised Expiry Date: N/A

Site Notice: 	N/A

Reason:

Press Advert: 	N/A

Reason:

Relevant Site History

No relevant site history.

# Consultations

Not applicable.

## Planning Policy

National Planning Policy

The Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended)

Appraisal

In respect of the single storey side extension, Schedule 2, Part 1, Class A of the GPDO allows for the enlargement, improvement or other alteration of a dwellinghouse, subject to compliance with the following relevant criteria:

A.1 Development is not permitted by Class A if—

(a)permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class G, M, MA, N, P [PA] or Q of Part 3 of this Schedule (changes of use);

-This dwelling has not been subject to a change of use.

(b)as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse);

-The proposed side extension would not exceed 50% of the total ground covered.

(c)the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse;

- The side extension will not exceed the height of the highest part of the roof of the existing house.

(d)the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse;

-The height of the eaves of the single storey side extension would not exceed the height of the eaves of the existing dwelling.

(e)the enlarged part of the dwellinghouse would extend beyond a wall which—

(i)forms the principal elevation of the original dwellinghouse; or

(ii)fronts a highway and forms a side elevation of the original dwellinghouse;

- The proposed extension extends from a side elevation of the original dwellinghouse which fronts a highway. As such, the proposal is contrary to ii) above.

(f)subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and—

(i)extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached dwellinghouse, or 3 metres in the case of any other dwellinghouse, or

(ii)exceed 4 metres in height;

- Not applicable; the extension is to the side of the house.

(g) for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and—

(i)extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, or

(ii)exceed 4 metres in height;

- Not applicable; the extension is to the side of the house.

(h)the enlarged part of the dwellinghouse would have more than a single storey and—

(i)extend beyond the rear wall of the original dwellinghouse by more than 3 metres, or

(ii)be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall of the dwellinghouse;

-Not applicable. The application is for a single storey side extension.

(I)the enlarged part of the dwellinghouse would be within 2 metres of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3 metres;

- The height of the eaves of the single storey side extension would be 2.65m.

(j)the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would—

(i)exceed 4 metres in height,

(ii)have more than a single storey, or

(iii)have a width greater than half the width of the original dwellinghouse; or

- The proposed extension would not be more than 4m high, is single-storey in form and has a width of 3m which is less than half of the existing width of house.

[(ja) any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceeds or would exceed the limits set out in sub-paragraphs (e) to (j);]

-Not applicable.

(k)it would consist of or include—

(i)the construction or provision of a verandah, balcony or raised platform,

(ii)the installation, alteration or replacement of a microwave antenna,

(iii)the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or

(iv)an alteration to any part of the roof of the dwellinghouse; or

-The proposed extension would not consist of or include any of the above-mentioned features numbered (i) to (iv).

(l) the dwellinghouse is built under Part 20 of this Schedule (construction of new dwellinghouses).

-Not applicable.

Recommendation:

Refuse

The application is NOT permitted by Schedule 2, Part 1, Class A of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended) as the proposed development would not meet A.1 (e)(ii) in that the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse which fronts a highway.

Notes to Applicant

This decision has been based upon the following plans received:

- Location Plan: 2022_06_03 -00

- Site Plan: 2022_06_03 -01

- Existing Ground Floor Plan: 2022_06_03 -02 Rev A

- Existing Elevations: 2022_06_03 -03 Rev A

- Proposed Ground Floor Plan: 2022_06_03 -04 Rev B

- Proposed Elevations: 2022_06_03 -05 Rev B

| Officer | James Bridgman | DC |  | LPA | M Hodgson |
| --- | --- | --- | --- | --- | --- |
| Date | 05/10/2022 | Date |  | Date | 05.10.22 |