|  |  | From | Phil Mason |  |
| --- | --- | --- | --- | --- |
|  |  |  | ## Highway Design |  |
|  |  |  | Place Directorate |  |
|  |  | Phone | 07929 862689 |  |
|  |  | Email | Phil.mason@salford.gov.uk |  |
| To | Danielle Sammy | Place Directorate |  |  |
| Copy to | Planning.contact@salford.gov.uk | Place Directorate |  |  |
| # Date | 31 October 2022 | Reference |  | 2280310pm02EAHankinsonWay |
| Subject | ## CONSULTATION RESPONSE (AIR QUALITY, NOISE AND LAND CONTAMINATION) |  |  |  |

| ## Application Number: | ## 22/80310/FUL |
| --- | --- |
| ## Proposal: | ## Change Of use from shop (use class E(a) to takeaway (use class Sui Generis) with the installation of stainless steel spiral canopy flue |
| ## Location: | ## 35A Hankinson Way Salford M6 5JA |

Thank you for consulting me on planning application 22/80310/FUL.

Air Quality

There are no objections with respect to air quality.

Noise & Odours

The application site is located in an existing retail area of Salford, albeit in an area with no existing late night economy.  Opening hours are proposed until 01:00 Monday to Saturday, with 22:00 on Sundays, and an external extract outlet is proposed.

There are residential receptors located above the premises, albeit set back to the north east in a high rise above the shopping precinct.  There is potential for noise from the extract system and people coming and going, together with cooking odours to have an impact on residential properties.

The applicant has submitted details of the extract system (drawing HFS-10-06-02) which includes acoustic silencers and details of the proposed filtration.  The filtrations system proposed includes grease baffle filters, fine filtration, carbon filtration and details of the extract velocity to ensure adequate dispersion of cooking odours.

The proposed filtration is, in my experience, in line with similar systems in use throughout the City for hot food takeaways located in close proximity to residential premises, and provided it is maintained in accordance with manufacturers requirements will be suitable to adequately filter cooking odours.

It is also noted general prevailing SE direction winds will carry any residual odours away from the residential properties.

With respect to noise, the details show 2 No. in-line acoustic silencers within the flu, which again is typical of similar installations across the City.  With respect to noise from customers coming and going, particularly at night, the front of the premises is overhung by a parapet which will screen noise from the front of the premises from the residential properties.

Based on the above I am satisfied the proposal is unlikely to have a significant adverse impact on health, quality of life and amenity residents can expect for the area, and I have no objections to the application subject to the following conditions;

- The premises shall only be open for business or trade between the hours of

  - Monday to Saturday: 10:00 – 01:00

  - Sunday and public holidays: 10:00 – 22:00

- The kitchen extract, including acoustic silencers and filtration system shall be installed in accordance with submitted drawing HFS-10-06-02.

- The kitchen extract system including filtration shall be in operation at all times during food preparation and shall be maintained in accordance with manufacturer’s instructions at all times.

- The noise rating level (dB LAr,Tr) from external plant and equipment associated with the development, when operating simultaneously, shall not exceed the typical background noise level (LA90,T) by more than -5 dB at any time when measured at the boundary of the nearest noise sensitive premises.  Noise measurements and assessments shall be carried out according to BS 4142:2014+A1:2019 "Methods for rating and assessing industrial and commercial sound".  ‘T’ refers to any 1 hour period between 07.00hrs and 23.00hrs and any 15 minute period between 23.00hrs and 07.00hrs

Land Contamination

There are no objections with respect to land contamination.

Should you require any further information please do not hesitate to contact me.

Phil Mason MSc, PGDip Acoustics, MIEnvSc, MIAQM, MCIEH

Environmental Consultant | Salford City Council

T: 07929 862689 | E-mail: Phil.Mason@salford.gov.uk